# ZombieVerter VCU — Parameter Reference

Auto-generated from param_prj.h (STM32-VCU-DP, release/230A.DAALPO.2)

PARAM entries are user-configurable settings stored in flash. VALUE entries are read-only runtime measurements.

# Configurable Parameters (PARAM)

## SETUP — Hardware Selection

These parameters select which inverter, vehicle CAN class, gear lever, BMS, charger interface and current shunt are connected. Set these first before tuning anything else.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| Inverter | mode | 0 | 8 | 0 | Inverter type: 0=None, 1=Leaf Gen1, 2=GS450H, 3=UserCAN, 4=OpenInverter, 5=Prius Gen3, 6=Outlander, 7=GS300H, 8=RearOutlander |
| Vehicle | mode | 0 | 8 | 0 | Vehicle CAN class: 0=BMW E46, 1=BMW E6x+, 2=Classic, 3=None, 5=BMW E39, 6=VAG, 7=Subaru, 8=BMW E31 |
| GearLvr | mode | 0 | 4 | 0 | Gear lever type: 0=None, 1=BMW F30, 2=JLR G1, 3=JLR G2, 4=BMW E65. Determines direction from CAN gear lever rather than digital inputs. |
| Transmission | mode | 0 | 1 | 0 | Transmission type: 0=Manual, 1=Auto (e.g. GS450H automatic) |
| interface | mode | 0 | 4 | 0 | Charge inlet interface: 0=Unused, 1=i3LIM, 2=CHAdeMO, 3=CPC, 4=Foccci |
| chargemodes | mode | 0 | 6 | 0 | Charger module: 0=Off, 1=Ext Digital, 2=Volt/Ampera, 3=Leaf PDM, 4=TeslaOI, 5=Outlander, 6=Elcon |
| BMS_Mode | mode | 0 | 6 | 0 | BMS type: 0=Off, 1=SimpBMS, 2=TI Daisychain Single, 3=TI Daisychain Dual, 4=Leaf BMS, 5=Renault Kangoo 33, 6=EMUS |
| ShuntType | mode | 0 | 3 | 0 | Current shunt/sensor: 0=None, 1=ISA shunt, 2=SBOX, 3=VAG |
| MotActive | mode | 0 | 3 | 0 | Active motor selection (GS450H dual motor): 0=MG1+MG2, 1=MG1 only, 2=MG2 only, 3=Blended MG2+MG1 |
| SpeedFactor | ratio | 0 | 500 | 84 | Inverter revolutions per wheel revolution. Used to calculate vehicle speed from motor RPM. |
| IMD_Type | mode | 0 | 2 | 0 | Insulation Monitoring Device type: 0=None, 1=Simple Bender |

## CAN Bus Assignment

Each subsystem can be assigned to CAN1 (0) or CAN2 (1). Distribute load to avoid bus saturation.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| InverterCan | bus | 0 | 1 | 0 | CAN bus for inverter communication |
| VehicleCan | bus | 0 | 1 | 1 | CAN bus for vehicle (dashboard, body CAN) |
| ShuntCan | bus | 0 | 1 | 0 | CAN bus for current shunt/sensor |
| LimCan | bus | 0 | 1 | 0 | CAN bus for limiter messages |
| ChargerCan | bus | 0 | 1 | 1 | CAN bus for charger module |
| BMSCan | bus | 0 | 1 | 1 | CAN bus for BMS |
| OBD2Can | bus | 0 | 1 | 0 | CAN bus for OBD2 interface |
| CanMapCan | bus | 0 | 1 | 0 | CAN bus for custom CAN map |
| DCDCCan | bus | 0 | 1 | 1 | CAN bus for DC-DC converter |
| HeaterCan | bus | 0 | 1 | 1 | CAN bus for heater |
| CAN3Speed | mode | 0 | 2 | 0 | Third CAN bus speed: 0=33.3k, 1=500k, 2=100k |

## THROTTLE — Drive & Protection

Throttle calibration, ramp rates, RPM limits and protection thresholds. Tune potmin/potmax first with the actual pedal before setting anything else.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| potmin | dig | 0 | 4095 | 0 | Throttle channel 1 ADC value at zero pedal position |
| potmax | dig | 0 | 4095 | 4095 | Throttle channel 1 ADC value at full pedal position |
| pot2min | dig | 0 | 4095 | 4095 | Throttle channel 2 ADC value at zero (inverted channel typical) |
| pot2max | dig | 0 | 4095 | 4095 | Throttle channel 2 ADC value at full |
| potmode | mode | 0 | 1 | 0 | Throttle input mode: 0=Single channel, 1=Dual channel (redundant/sanity check) |
| dirmode | mode | 0 | 4 | 1 | Direction input mode: 0=Button, 1=Switch, 2=Button Reversed, 3=Switch Reversed, 4=Default Forward |
| reversemotor | on/off | 0 | 1 | 0 | Invert motor rotation direction without rewiring |
| throtramp | %/10ms | 1 | 100 | 10 | Throttle ramp rate — limits how fast throttle command can increase. Lower = smoother but less responsive. |
| throtramprpm | rpm | 0 | 20000 | 20000 | RPM below which the throttle ramp rate applies. Above this, ramp is set to maximum. |
| throtmax | % | 0 | 100 | 100 | Maximum forward throttle output percentage |
| throtmin | % | -100 | 0 | -100 | Maximum regen/reverse throttle output percentage |
| throtmaxRev | % | 0 | 100 | 30 | Maximum throttle percentage when in reverse. Limits reverse speed. |
| throtdead | % | 0 | 50 | 10 | Throttle deadband percentage. Below this, throttle command is zero. |
| throtrpmfilt | rpm/10ms | 0.1 | 200 | 15 | RPM filter rate for throttle calculations |
| revlim | rpm | 0 | 20000 | 6000 | Motor RPM limit — inverter will be commanded to zero above this speed |
| revRegen | on/off | 0 | 1 | 0 | Enable regenerative braking when driving in reverse |
| regenrpm | rpm | 100 | 10000 | 1500 | RPM above which regen becomes active |
| regenendrpm | rpm | 100 | 10000 | 100 | RPM below which regen tapers off to zero |
| regenmax | % | -35 | 0 | -10 | Maximum regen torque percentage (negative = braking) |
| regenBrake | % | -35 | 0 | -10 | Additional regen torque applied when brake pedal is pressed |
| regenramp | %/10ms | 0.1 | 100 | 1 | Ramp rate for regen torque changes |
| RegenBrakeLight | % | -100 | 0 | -15 | Regen level (%) at which the brake light output activates |
| udcmin | V | 0 | 1000 | 450 | DC bus undervoltage threshold — drive is cut below this voltage |
| udclim | V | 0 | 1000 | 520 | DC bus overvoltage threshold — system shuts down above this voltage |
| idcmax | A | 0 | 5000 | 5000 | Maximum DC current draw (motoring) limit |
| idcmin | A | -5000 | 0 | -5000 | Maximum DC current return (regen) limit |
| tmphsmax | °C | 50 | 150 | 85 | Inverter heatsink temperature limit — throttle derated above this |
| tmpmmax | °C | 70 | 300 | 300 | Motor temperature limit — throttle derated above this |
| bmslimhigh | % | 0 | 100 | 50 | BMS high-side throttle limit — max drive torque allowed by BMS |
| bmslimlow | % | -100 | 0 | -1 | BMS low-side throttle limit — max regen allowed by BMS |

## CRUISE CONTROL

Cruise control speed stepping and ramp behaviour. Also doubles as regen level adjustment when cruise is off.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| cruisestep | rpm | 1 | 1000 | 200 | Speed increment/decrement per cruise control button press |
| cruiseramp | rpm/100ms | 1 | 1000 | 20 | Ramp rate for cruise control speed changes and resume |
| regenlevel | 0–3 | 0 | 3 | 2 | Regen preset level when adjusting with cruise buttons (0=off, 3=max) |

## CONTACTOR / STATUS OUTPUTS

Precharge completion voltage and dashboard indicator output configuration.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| udcsw | V | 0 | 1000 | 330 | DC bus voltage at which precharge is considered complete and main contactor closes |
| cruiselight | on/off | 0 | 1 | 0 | Enable cruise control active indicator output |
| errlights | bitmask | 0 | 255 | 0 | Error indicator outputs: 0=Off, 4=EPC light, 8=Engine light |

## BMS — Battery Management Limits

Configurable safety thresholds for BMS monitoring. The BMS implementation sets the VALUE entries (BMS_Vmin etc.) from CAN; these PARAM entries define the limits at which action is taken.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| BMS_Timeout | sec | 1 | 120 | 10 | Seconds without a BMS CAN message before a fault is raised |
| BMS_VminLimit | V | 0 | 10 | 3.0 | Cell undervoltage alarm threshold |
| BMS_VmaxLimit | V | 0 | 10 | 4.2 | Cell overvoltage alarm threshold |
| BMS_TminLimit | °C | -100 | 100 | 5 | Minimum cell temperature alarm threshold |
| BMS_TmaxLimit | °C | -100 | 100 | 50 | Maximum cell temperature alarm threshold |

## CHARGER

Onboard and DC fast charge parameters. Voltspnt/Pwrspnt control the onboard charger; CCS_* parameters control DC fast charge via the Foccci/CPC interface.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| BattCap | kWh | 0.1 | 250 | 22 | Battery pack capacity — used for SOC and energy accounting |
| Voltspnt | V | 0 | 1000 | 395 | Onboard charger target voltage setpoint |
| Pwrspnt | W | 0 | 12000 | 1500 | Onboard charger target power setpoint |
| IdcTerm | A | 0 | 150 | 0 | Charge termination current — charging stops when current falls below this |
| CCS_ICmd | A | 0 | 150 | 0 | CCS DC fast charge current command sent to EVSE |
| CCS_ILim | A | 0 | 350 | 100 | CCS maximum current limit accepted from EVSE |
| CCS_SOCLim | % | 0 | 100 | 80 | SOC level at which CCS charging is terminated |
| SOCFC | % | 0 | 100 | 50 | Full charge SOC target for timer-controlled charging |
| Chgctrl | mode | 0 | 2 | 0 | Charge control: 0=Enable, 1=Disable, 2=Timer controlled |
| ChgAcVolt | Vac | 0 | 250 | 240 | AC supply voltage — used for efficiency and power calculations |
| ChgEff | % | 0 | 100 | 90 | Charger efficiency percentage — used to calculate AC draw from DC power |
| ConfigFoccci | on/off | 0 | 1 | 0 | Trigger Foccci CCS interface auto-configuration over CAN |

## DC-DC CONVERTER

12V DC-DC converter configuration.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| DCdc_Type | mode | 0 | 1 | 0 | DC-DC converter type: 0=None, 1=Tesla Gen2 |
| DCSetPnt | V | 9 | 15 | 14 | 12V rail voltage target setpoint sent to the DC-DC converter |

## HEATER

High-voltage cabin and battery heater configuration.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| Heater | mode | 0 | 3 | 0 | Heater type: 0=None, 1=Ampera, 2=VW, 3=Outlander CAN |
| Control | mode | 0 | 2 | 0 | Heater control: 0=Disable, 1=Enable (always on), 2=Timer controlled |
| HeatPwr | W | 0 | 6500 | 0 | Heater power setpoint in watts |
| HeatPercnt | % | 0 | 100 | 0 | Heater power as a percentage of maximum |

## CLOCK — Scheduled Charging & Preheating

Real-time clock settings and departure-time scheduling for charging and cabin preheating.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| Set_Day | 0–6 | 0 | 6 | 0 | Set RTC day of week (0=Sunday) |
| Set_Hour | H | 0 | 23 | 0 | Set RTC hour |
| Set_Min | min | 0 | 59 | 0 | Set RTC minute |
| Set_Sec | sec | 0 | 59 | 0 | Set RTC second |
| Chg_Hrs | H | 0 | 23 | 0 | Scheduled charge start hour |
| Chg_Min | min | 0 | 59 | 0 | Scheduled charge start minute |
| Chg_Dur | min | 0 | 600 | 0 | Scheduled charge duration in minutes |
| Pre_Hrs | H | 0 | 59 | 0 | Scheduled preheat start hour |
| Pre_Min | min | 0 | 59 | 0 | Scheduled preheat start minute |
| Pre_Dur | min | 0 | 60 | 0 | Scheduled preheat duration in minutes |

## I/O PIN FUNCTIONS

Each digital output, signal lamp and PWM output can be assigned a function. Analogue inputs can be assigned as proximity pilot or brake vacuum sensor.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| Out1Func | mode | 0 | 20 | 6 | Digital output 1 function (default: RunIndication) |
| Out2Func | mode | 0 | 20 | 7 | Digital output 2 function (default: WarnIndication) |
| Out3Func | mode | 0 | 20 | 3 | Digital output 3 function (default: HeaterEnable) |
| SL1Func | mode | 0 | 20 | 0 | Signal lamp 1 function |
| SL2Func | mode | 0 | 20 | 0 | Signal lamp 2 function |
| PWM1Func | mode | 0 | 20 | 0 | PWM output 1 function |
| PWM2Func | mode | 0 | 20 | 4 | PWM output 2 function (default: RunIndication) |
| PWM3Func | mode | 0 | 20 | 2 | PWM output 3 function (default: OBCEnable) |
| GP12VInFunc | mode | 0 | 13 | 12 | General purpose 12V input function |
| HVReqFunc | mode | 0 | 13 | 12 | HV request input function |
| PB1InFunc | mode | 0 | 13 | 12 | Pushbutton input 1 function |
| PB2InFunc | mode | 0 | 13 | 12 | Pushbutton input 2 function |
| PB3InFunc | mode | 0 | 13 | 12 | Pushbutton input 3 function |
| GPA1Func | mode | 0 | 2 | 0 | Analogue input 1: 0=None, 1=Proximity Pilot, 2=Brake Vacuum Sensor |
| GPA2Func | mode | 0 | 2 | 0 | Analogue input 2: 0=None, 1=Proximity Pilot, 2=Brake Vacuum Sensor |
| PumpPWM | mode | 0 | 1 | 0 | PWM output type: 0=GS450H oil pump, 1=Tacho output |
| ppthresh | dig | 0 | 4095 | 2500 | Proximity pilot detection ADC threshold |
| BrkVacThresh | dig | 0 | 4095 | 2500 | Brake vacuum sensor low-vacuum ADC threshold |
| BrkVacHyst | dig | 0 | 4095 | 2500 | Brake vacuum sensor hysteresis ADC value |
| DigiPot1Step | dig | 0 | 255 | 0 | Digital potentiometer 1 step value |
| DigiPot2Step | dig | 0 | 255 | 0 | Digital potentiometer 2 step value |
| FanTemp | °C | 0 | 100 | 40 | Temperature at which the cooling fan output activates |
| TachoPPR | PPR | 0 | 100 | 2 | Tacho output pulses per motor revolution |

## SHUNT

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| IsaInit | on/off | 0 | 1 | 0 | Send ISA shunt initialisation sequence on startup |

## RAW PWM — Timer 3

Low-level Timer 3 registers for the three PWM channels. Used for control pilot, oil pump and other PWM functions. Period = (Prescaler+1) × (Period+1) / 72MHz.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| Tim3_Presc |  | 1 | 72000 | 719 | Timer 3 prescaler value (default gives 100kHz base clock) |
| Tim3_Period |  | 1 | 100000 | 7200 | Timer 3 period register (default gives 10kHz PWM) |
| Tim3_1_OC |  | 1 | 100000 | 3600 | Timer 3 channel 1 compare value (default 50% duty) |
| Tim3_2_OC |  | 1 | 100000 | 3600 | Timer 3 channel 2 compare value (default 50% duty) |
| Tim3_3_OC |  | 1 | 100000 | 3600 | Timer 3 channel 3 compare value (default 50% duty) |
| CP_PWM | % | 1 | 100 | 10 | Control pilot PWM duty cycle percentage |

## LEXUS / GS450H

Parameters specific to the Toyota/Lexus GS450H hybrid drivetrain.

| Name | Unit | Min | Max | Default | Description |
| --- | --- | --- | --- | --- | --- |
| Gear | mode | 0 | 3 | 0 | Target gear for GS450H transmission: 0=Low, 1=High, 2=Auto, 3=HighFwdLowRev |
| OilPump | % | 0 | 100 | 50 | GS450H transmission oil pump PWM duty cycle |

# Runtime Values (VALUE)

## Runtime Values — Electrical

Read-only live measurements from inverter, shunt and BMS.

| Name | Unit | Description |
| --- | --- | --- |
| udc | V | DC bus voltage (primary measurement) |
| udc2 | V | DC bus voltage (secondary — e.g. battery side with SBOX) |
| udc3 | V | DC bus voltage (tertiary) |
| deltaV | V | Maximum voltage imbalance between udc, udc2, udc3 |
| INVudc | V | DC voltage as reported by the inverter over CAN |
| idc | A | DC bus current (positive = motoring, negative = regen) |
| power | kW | Instantaneous power (udc × idc) |
| KWh | kWh | Cumulative energy used since last reset |
| AMPh | Ah | Cumulative charge used since last reset |
| SOC | % | State of charge |
| uaux | V | Auxiliary 12V rail voltage (measured via on-board divider) |
| U12V | V | 12V supply voltage (from DC-DC or external sensor) |
| I12V | A | 12V supply current |

## Runtime Values — Drive

| Name | Unit | Description |
| --- | --- | --- |
| speed | rpm | Motor speed |
| Veh_Speed | kph | Calculated vehicle speed (speed ÷ SpeedFactor) |
| torque | dig | Torque command sent to inverter (raw units) |
| potnom | % | Normalised throttle command after all limiting (-100 to +100%) |
| pot | dig | Throttle pedal channel 1 raw ADC value |
| pot2 | dig | Throttle pedal channel 2 raw ADC value |
| potbrake | dig | Brake pedal/sensor raw ADC value |
| brakepressure | dig | Brake pressure sensor ADC value |
| dir |  | Current direction: -1=Reverse, 0=Neutral, 1=Drive, 2=Park |
| opmode |  | Operating mode: 0=Off, 1=Run, 2=Precharge, 3=PchFail, 4=Charge |
| status | bitmask | System status flags: UdcLow, UdcHigh, UdcBelowUdcSw, UdcLim, EmcyStop, MProt, PotPressed, TmpHs, WaitStart |
| TorqDerate |  | Torque derate reason: None, UDClimLow, UDClimHigh, IDClimLow, IDClimHigh, TempLim |
| FrontRearBal | % | Front/rear torque balance (dual motor systems) |
| cruisespeed | rpm | Active cruise control target speed |
| cruisestt |  | Cruise control state: None, On, Disable, Set, Resume |

## Runtime Values — Temperatures

| Name | Unit | Description |
| --- | --- | --- |
| tmphs | °C | Inverter heatsink temperature |
| tmpm | °C | Motor temperature |
| tmpaux | °C | Auxiliary temperature sensor |
| tmpheater | °C | High-voltage heater temperature |
| ChgTemp | °C | Charger temperature |

## Runtime Values — Digital Inputs

| Name | Unit | Description |
| --- | --- | --- |
| din_forward | on/off | Forward direction input (physical pin or CAN IO) |
| din_reverse | on/off | Reverse direction input (physical pin or CAN IO) |
| din_brake | on/off | Brake input (physical pin or CAN IO) |
| din_cruise | on/off | Cruise control input (CAN IO only) |
| din_start | on/off | Start/enable input (physical pin or CAN IO) |
| din_bms | on/off | BMS OK signal (CAN IO only) |
| din_12Vgp | on/off | General purpose 12V input state |
| handbrk | on/off | Handbrake/park brake input |
| Gear1 | on/off | Gear sensor input 1 |
| Gear2 | on/off | Gear sensor input 2 |
| Gear3 | on/off | Gear sensor input 3 |
| GearFB |  | Gear feedback from transmission: Low, High, Auto, HighFwdLowRev |
| T15Stat | on/off | Terminal 15 (ignition) status |
| InvStat | on/off | Inverter ready/run status |
| VehLockSt | on/off | Vehicle lock state |
| canio | bitmask | CAN IO input state: Cruise, Start, Brake, Fwd, Rev, BMS |
| canctr | dig | CAN message counter (increments each cycle for watchdog purposes) |
| PPVal | dig | Proximity pilot raw ADC value |
| BrkVacVal | dig | Brake vacuum sensor raw ADC value |
| cpuload | % | CPU load percentage |
| lasterr |  | Last error message string |
| version |  | Firmware version string |

## Runtime Values — BMS

| Name | Unit | Description |
| --- | --- | --- |
| BMS_Vmin | V | Lowest cell voltage reported by BMS |
| BMS_Vmax | V | Highest cell voltage reported by BMS |
| BMS_Tmin | °C | Lowest cell temperature reported by BMS |
| BMS_Tmax | °C | Highest cell temperature reported by BMS |
| BMS_Tavg | °C | Average cell temperature (Leaf BMS) |
| BMS_ChargeLim | A | Maximum charge current allowed by BMS |
| BMS_MaxInput | kW | Maximum power the battery can accept (charging) |
| BMS_MaxOutput | kW | Maximum power the battery can deliver (discharging) |
| BMS_MaxCharge | W | Maximum charge power in watts |
| BMS_Isolation | Ohm | Battery isolation resistance (Kangoo/STW BMS) |
| BMS_IsoMeas | mV | Isolation measurement voltage (Leaf BMS) |

## Runtime Values — Charging & CCS

| Name | Unit | Description |
| --- | --- | --- |
| chgtyp |  | Charge type detected: Off, AC, DCFC |
| CableLim | A | Maximum current from charge cable (proximity pilot decode) |
| PilotLim | A | Maximum current from control pilot PWM duty cycle |
| PlugDet | on/off | Charge plug detected |
| PilotTyp |  | Control pilot type: Absent, ACStd, ACchg, Error, CCS_Not_Rdy, CCS_Rdy, Static |
| hvChg | on/off | High-voltage charging active |
| CCS_I_Avail | A | CCS maximum current available from EVSE |
| CCS_V_Avail | V | CCS maximum voltage available from EVSE |
| CCS_I | A | CCS actual charge current |
| CCS_Ireq | A | CCS current request sent to EVSE |
| CCS_V | V | CCS actual charge voltage |
| CCS_V_Min | V | CCS minimum voltage from EVSE |
| CCS_V_Con | V | CCS contracted/agreed voltage |
| CCS_COND |  | CCS condition state |
| CCS_State | s | CCS state machine timer |
| CCS_Contactor | on/off | CCS DC contactor state |
| CP_DOOR |  | Charge port door state: Closed, Open, Error, Invalid |
| ChgT | min | Charge timer remaining |
| AC_Volts | V | AC supply voltage measured |
| AC_Amps | A | AC supply current measured |
| HeatReq | on/off | Heater heat request active |
| udcheater | V | Heater DC supply voltage |
| powerheater | W | Heater power consumption |

## Runtime Values — Clock

| Name | Unit | Description |
| --- | --- | --- |
| Day |  | Current day of week (0=Sunday) |
| Hour | H | Current hour |
| Min | M | Current minute |
| Sec | S | Current second |

## Runtime Values — IMD (Insulation Monitoring)

| Name | Unit | Description |
| --- | --- | --- |
| ImdFault | on/off | Insulation fault detected |
| ImdState |  | IMD state: IMD_OFF, IMD_NORMAL, IMD_MEASUREMENT_ERROR |
| CanAct | on/off | CAN bus activity detected |